Sheet (4)

- In a nonmagnetic medium (o and ) if the electric field intensity of a plane wave is given by:

Determine and calculate the following:

a) r and .

b) The time-average power density of the wave,     Pdav               .

c) The total time-average power, Ptav   crossing a normal area, A = 100 cm2

- Consider a plane wave normally incident on a half-space of copper ( = 5.813x107 S/m, µr = 1 and εr = 1). If wave frequency, f = 1 GHz, compute:

- The propagation constant or wave number, ..

- The intrinsic impedance, η.

- The skin depth, δs for the conductor.

- The reflection coefficient, Γ.

- The transmission coefficient, T.

- Consider a plane wave normally incident from free space on a half-space of a medium 1 with parameters (µr = 1 and εr =2.5) as shown in Fig. 1. If the wave frequency, f = 1 GHz and the incident electric field in the phasor form, for z < 0 is:

where E+ = 10 mV/m, find the phasor and instantaneous values of the incident wave (Ei   ,Hi    ), the reflected wave (  Er  ,  Hr  ) and the transmitted wave   ( Et   ,  Ht  ).

- Consider a plane wave normally incident from the free space on a half-space of a medium with parameters (σ = 0, µ = µo, ε = 3.3 εo). If the wave frequency f = 1.5 GHz and the incident electric field intensity in the phasor form, for z < 0 is:

Determine and calculate the phasor and instantaneous form value of the following:

a) The incident magnetic field intensity H, and the incident average power density, Pidav

b) The reflected wave (Er   , Hr  ) and the reflected average power density,      .

c) The transmitted wave ( Et  ,  Ht ) and the transmitted average  power density,     .