Part 2 - Sheet 3

1  The magnetic field intensity is given in the square region x=0,  0.5 < 𝑦 < 1  ,  1 < 𝑧 <1.5 by 𝐻 = 𝑧2𝑎𝑥+ 𝑥3𝑎𝑦+ 𝑦4𝑎𝑧 A/m.

a)  Evaluate ∮ 𝐻. 𝑑𝐿,  about the perimeter of the square region.

b)  Find ∇ × 𝐻

c)  Calculate (∇ × 𝐻)𝑥 at the center of the region.

2  A wooden rail Toroid with internal radius, a = 25 cm, external radius, b = 45 cm number of turns, N =500 and carries current, I = 1 mA. Find the magnetic field intensity and the magnetic flux density at the center of this Toroid if µr = 1 for wood.

3  A Toroid of a ring-shaped coil, shown in Fig. 3.18, whose parameters are: the mean radius rav = 10 cm, winding is uniformly distributed and contains number of turns, N =200 turns and wound on a wooden core with diameter, d = 2 cm. If it carries current, I = 1 mA do the following:

(a) Derive an expression for the self-inductance L.

(b) Calculate L for the given parameters if µr = 1 for wood.

4  A circular cross-section conductor of radius, r = 2 mm carries an ac current, ic of                           ic = 2.5 sin (5 x 108 t) µA. What is the amplitude of the displacement current density if the conductivity of the materials, = 35 MS/m and the relative permittivity, r= 1?

5  In a material for which the conductivity of the materials, = 5 S/m and the relative permittivity, r= 1, the electric field intensity, E = = 250 sin (1010 t) V/m. Find the conduction and displacement current densities and the frequency at which they have equal magnitudes.

6  Let µ = 3 x 10-5 H/m, є = 1.2x 10-10 F/m, and σ = 0 every where. If H =2 cos(1010t -βx) âz A/m, use Maxwell's equations to obtain expressions for B, D, E, and  β.

7  A certain material has σ = 0 and єR =1. if H = 4sin(106t – 0.01z)ay A/m, make use of Maxwell's equations to find µR and E(z, t).