Electrical Engineering Department

Digital Signal Processing                                                 4th Year.

Midterm Exam                                                                 Allowed Time: 90 min.

Date: November-2013.

-----------------------------------------------------------------------------------------------------------------------------------------

Answer the following questions:

- Signal Manipulation

- Determine whether or not the signals below are periodic and for each periodic signal determine the fundamental period:

- Given the sequence

X(n) = (3- 0.5n) (u(n) – u(n-3))        Sketch

- Y(n) = x(3-3n)

- Y(n) = x(n2  - 2n +1)

2. Discrete Time Systems

- Check system linearity

- Y(n) = 3x(n-2) + 2x(n-1) + x(n) – 1

- Y(n) = x(n) Cos (nπ/6)

- For a system with its response

hk(n) = (n-k) u(n-k)

Determine whether or not the system is shift invariant

- Check the system stability

3. Convolution

Convolve

- X(n) = 2 [u(n) – u(n-4)]   with

h(n) = 0.5 Cos (nπ/2) [u(n+4) – u(n-3)] and sketch y(n)

- X(n) = (1/6)n-6  u(n)   with

h(n) = (1/3)n u(n-3)

4. Frequency Response

- Let h(n) be the unit sample response of LSI system find the frequency response  if :

h(n) = (1/5)n+4 u(n-4)

- Find the DTFT for

X(n) =                      (1/2)n                        n = 0,2,4,..

0                            otherwise