Electrical Engineering Department

Digital Signal Processing                                                 4th Year.

Midterm Exam                                                                 Allowed Time: 75 min.

Date: November-2014.

-----------------------------------------------------------------------------------------------------------------------------------------

Answer the following questions: [25 Marks]

- Signal Manipulation

- Find the even and odd parts of the following signals                  [4 Marks]:

- x1(n) = u(n).

- x2(n) = an u(n).

- A sinusoidal signal x(t) = Cos(wt) is sampled at the Nyquest rate starting at (wt = 0) (the signal frequency is 10 kHz) find

[4 Marks]:

  - Sampling frequency.

  - Draw x(n) [at least 2 cycles].

  - Draw the delayed version of x(n) by 3 samples.

  - Draw the compressed version of x(n) by a factor of 2.

2. Discrete Time Systems  [6 Marks]

- Determine if the system below is linear or not.

y(n) = 5 x(n+2) + 2 x(n+1) + 3x(n) + 1

- Determine whether or not the following system is shift-invariant.

y(n) = x(n) + x(n-1) + x(n-2)

3. Convolution  [11 Marks]

Convolve

- x(n) = 2 Cos (nπ/2) [u(n) – u(n-4)]   with

h(n) = 0.5 n  [u(n) – u(n-3)] and sketch y(n)

- x(n) = (0.9)n  u(n)   with

h(n) = n u(n)