Electrical Engineering Department                                    4th Year.

Digital Signal Processing                                                         Allowed Time: 75 min.

Midterm Exam                                                                       Date: November-2015.

-----------------------------------------------------------------------------------------------------------------------------------------

Answer the following questions: [25 Marks]

- Signal Manipulation

- Determine whether or not the signals below are periodic and, for each signal that is periodic, determine the fundamental period [4 Marks]:

- Given the sequence x(n) = (10 -2 n)[u(n) - u(n - 5 ) ] , make a sketch of  [4 Marks]:

  - y1(n) = x(2n - 2)

  - y 2(n) = x(n2 - 2n  )

2. Discrete Time Systems  [6 Marks]

- Determine if the system below is linear or not.

y(n) = 6 x(n) +[ x(n+1) + x(n-1)]/ x(n)

- Determine whether or not the following system is shift-invariant.

y(n) = x(n)u(n) + x(n-1)u(n) + x(n-2)u(n)

3. Convolution and DTFT [11 Marks]

Convolve  [7 Marks]

- x(n) =( 1+0.5 Cos (nπ/2)) [u(n) – u(n-4)]   with

h(n) = 2 n  [u(n+2) – u(n-3)] and sketch y(n)

B.  Find the DTFT of each of the following sequence: [4 Marks]

x(n) = (1/2)n u(n-3)