BENHA UNIVERSITY                                                   Electric and magnetic properties of materials

# FACULTY OF ENGINEERING (SHOUBRA)                                 First Electrical Year (Power)

ELECTRICAL ENG. DEPARTMENT            28, January 2010                        Total Time: 3 hours

-------------------------------------------------------------------------------------------------------------------------------

(Page 1)

## Attempt the following questions: -

1.a) Write notes on each of the following items: -                                  ( 15 Marks)

- Piezoelectricity

- Stable chemical Compound

- How can you measure the dielectric constant Ɛr for a particular material?

- Neel temperature

- Remanent polarization Pr

1.b) For a single electron in an atom, draw a schematic representation of the frequency dependence of the real and imaginary parts (α´e , α´´e ) of the electronic polarizability.

(5 Marks)

1.c) Consider a parallel plate condenser with a lossy dielectric between them. The area of the plate is 0.5 m2, the distance between the two plates = 30 cm. At an angular frequency ω let the dielectric be characterized by a complex dielectric constant   εr =  εr' - j εr"

If the real part of the dielectric constant εr' = 6.68 and the loss tangent tan δ= 8 x 10-3 at an applied voltage v(t) = 1 x 106  cos  ω t  volt and a frequency f = 150 cycle/sec. Find :-

- The dielectric losses,                                                                                  ( 4 Marks)

-  The resultant current i(t) through the lossy condenser.                             ( 6 Marks)

2.a) How can you compare between the paramagnetic, ferromagnetic and antiferromagnetic materials?                                                                                                          ( 6 Marks)

For each type draw the relation between the reciprocal of the magnetic susceptibility 1/X and the temperature T at high temperature region.                                                   ( 4 Marks)

2.b) An electron describing a circular orbit of radius R = 2x10-15 meter rotates with ωo = 1.2 x 1012 radian/sec. Find:-

- The orbital magnetic dipole moment (μmo) ,                                                     ( 2 Marks)

- The angular momentum (Ma).                                                                           ( 2 Marks)

Suppose that, a magnetic flux density (B) increases to 2 Tesla, find:-

-  The Larmor angular frequency (ωL) ,                                                               ( 3 Marks)

- The induced magnetic dipole moment (μm induced) in Bohr magneton by this field.

( 3 Marks)

مع أمنياتي للجميع بالتوفيق والنجاح &                                                                             أ.د. محمد مؤنس

من فضلك اقلب الصفحه لمتابعة بقية أسئلة الجزء الثاني

1 / 2