| Benha University Mechanical Engineering Department First term (2014-2015) | Sheet No (2) | Faculty of Engineering-Shoubra 3rd year (Mechanical Power) Fluid Mechanics (3) |
| --- | --- | --- |

- Prove that the speed of sound in a perfect gas is given by : c =         then determine the speed of sound for the following cases

a- Air at 20         b- Hydrogen at 20  ,   = 1.41          c- Mercury at 20  ,    E=26.2 1010 N/m2

- start with the relation for stagnation pressure which is valid a perfect gas

Po = P

Expand the right side in a binomial series and evaluate the result for small   Mach numbers.

- Air at 15 oC and 1.013 bar flowing at velocity of 100 m/s.  Determine the stagnation pressure:

- Neglecting the compressibility.

- Taking the compressibility into consideration

Repeat the solution for V=1500 m/s and 3000 m/s and compare the results.

- An airplane files at an altitude of 10,000m where T=223 K and P=0.264 bar with a velocity of 800 Km/hr. Calculate: a) The maximum possible temperature on the airplane skin.

b) The maximum possible pressure on the airplane body.

c) The critical and maximum. Velocities of air relative to airplane.

- An aircraft files with 800 Km/hr at an altitude of 10,000 m Where: T=223 K, P=0.264 bar. The air is reversibly compressed in an inlet diffuser. If the Mach number at the exit of the diffuser is 0.36, determine the entry Mach number and the velocity, pressure and temperature of air at exit.

- A stream of air flowing in a duct has P=1.4 bar, m. =0.25 kg/s and M=0.6 at a section where the flow area is 6.5 cm2. Calculate the stagnation temperature of the air.

- Prove the following relations for isentropic flow:

- At a given point in the high-speed flow over an airplane wing, the local Mach number, pressure and temperature are 0.7, 0.9 atm, and 250 K, respectively. Calculate the values of Po, To, P*, T*, and a* at this point.

- Show that the Area-Velocity relation is given by:

- Carbon dioxide flows steadily through a varying cross-sectional area duct such as a nozzle at a mass flow rate of 3 kg/s. The carbon dioxide enters the duct at a pressure of 1400 kPa and 200°C with a low velocity, and it expands in the nozzle to a pressure of 200 kPa. The duct is designed so that the flow can be approximated as isentropic. Determine the density, velocity, flow area, and Mach number at exit.

- A point source is moving around the earth in an orbit of 13300km diameter at a speed of 1500km/hr. With respect to stationary coordinates, plot the wave front of the small disturbances produced by the source at time 0, t , 2t , etc. compute the time necessary for the sound wave to reach the earth, what is the Mach angle ? Assume that the earth is spherical (13000km diameter) , neglect its curvature and assume an average temperature of 60oC.

- An airplane is flying horizontally at an altitude of 7.5 km (where the temperature is 239K) and at a cruising speed of 2340Km/hr, find the distance that the plane has traveled from the head position of an observer at the ground when he hear the sound of the plane . Calculate also the time elapsed since the plane pass over the observer to the instant of hearing the plane sound. Also determine the Mach cone angle if it possible

- A supersonic aircraft is flying at an altitude of 3000m. An observer on the ground hears the noise generated by the aircraft which was at a distance of 2000m away horizontally, after 13s. Determine the Mach number, velocity of the aircraft, and air temperature at this altitude. Also determine the Mach cone angle if it possible.