| Benha University Mechanical Engineering Department First term (2015-2016) | Sheet No (6) | Faculty of Engineering-Shoubra 3rd year (Mechanical Power) Fluid Mechanics (3) |
| --- | --- | --- |

- Air flows adiabatically in a constant area duct. At the inlet of the duct, the velocity, pressure and temperature are 500 m/s, 200 kPa and 300 K, respectively. Assuming a duct length of 0.1 m, a duct diameter of 0.02 m and a friction factor of 0.002. Determine the Mach number, temperature and pressure at the exit of the duct. Also calculate the mass flow rate and frictional force on the duct walls.

- Air flows steadily and adiabatically through a pipe with a constant cross sectional area. Air is fed into the pope through an isentropic converging nozzle from a large tank where the pressure and temperature are 900 kPa and 300 K, respectively. The pressure at the inlet of the pipe is 750 kPa. If the temperature at the pipe exit is 275 K, determine the pressure at the pipe exit, the entropy increase and frictional force on the duct walls.

- A supersonic converging-diverging nozzle with an area ratio of 5 connects to an 2.5 m long, constant area rectangular duct. The duct is 0.2 x 0.1 m in cross section and has a friction factor of 0.005. The stagnation pressure of the air feeding the nozzle is 20 bar. Draw the T-s diagram and determine the receiver pressure and the entropy change through the system.

- A large chamber contains air at 65 bar and 400 K. The air passes through a converging nozzle and then into a constant area duct. The friction length of the duct is 1.067 and the Mach number at the duct exit is 0.96.

  - Draw the T-s diagram of the system.

  - Determine the conditions at the duct entrance.

  - What is the pressure in the receiver?

- A large chamber contains air at 8 bars and 300K the air enter a convergent – divergent nozzle with an area ratio of 2.4. A constant area duct is attached to the nozzle and a normal shock stands at the exit plane. The receiver pressure is 3 bars, assume the entire to be adiabatic and neglect friction in the nozzle. Compute the friction length of the duct.

- A convergent – divergent nozzle is designed to produce air at Mach number of 2.8 , the air enters the nozzle at stagnation conditions of 10 bars and 370 K , a constant area duct with a length 10 diameter is attached to the nozzle outlet , the friction factor in the duct is 0.005  compute

- Draw the T-S diagram for the system

- Compute the receiver pressure that would place a shock wave at the duct exit

- What is the total change in entropy for the system