| Benha University | Shoubra Faculty of Engineering |
| --- | --- |
| 1styear (2016-2017) | Electrical Engineering. Dept. |
| Thermodynamics Sheet No. (1) | Power engineering |

- What is the mass of air contained in a room 6m * 10 m * 4 m if the pressure is 100 Kpa and the temperature is 25 oC? Assume the air to be ideal gas.

- A tank has a volume of 0.5 m3 and contains 10 kg of an ideal gas having a molecular weight of 24 and temperature of 25oC what is the pressure

- A rigid tank contains 0.5 kg of oxygen (O2) initially at 4 Mpa and 180 K. the gas is cooled and the pressure drop to 3.3 Mpa. determine the volume of the tank in m3

- Consider the spherical balloon with diameter of 5 m; the local ambient conditions are 100 kpa and 20 oC.

- Calculate the mass in kg and the number of kmole of ambient air displaced by this balloon

- The balloon is filled with helium at 100 kpa and 20 oC. Calculate the mass in kg and the number of kmole of helium inside the balloon.

- Determine the specific properties data for (H2O) at the states indicated and locate each state carefully on  (T-v) and (P-v) diagram

- P=100Kpa and T=280oC.find v (m3/kg) and u (kj/kg)

- P= 0.15 Mpa and v= 0.9 m3/kg. find T (oC) and (kj/kg)

- P= 0.15 Mpa and x= 0.9  find (T, v, u , h  and  s)